# Customer name: JSC «Riga Electric Machine Building Works»

(AS «Rīgas elektromašīnbūves rūpnīca»)

Legal address: Ganibu Dambis 53, Riga, Latvia, LV-1005

Contact person: Vladimir Malgin, Chief Technologist

Telephone for contacts: + 371 27000891

E-mail: vladimir.malgin@rer.lv, dt@rer.lv

# Technical requirements for the supply of lathe No.1

# Requirements

| # № | # Parameter | # Requirement |
| --- | --- | --- |
| # 1. | Condition | New |
| # 2. | # Max turning diameter over bed | Not less than 400 mm |
| # 3. | # Max turning diameter over cross slide | # Not less than 220 mm |
| # 4. | # Max turning length | Not less than 1500 mm |
| # 5. | # Workpiece weight in a chuck | # Not less than 200 kg |
| # 6. | # Power supply | AC 3x400V, 50 Hz frequency |
| 7. | # Additional requirements | - A machine must provide parts production with grade not less than IT6 in accordance with ISO 286; - Auto lubrication system; - 3 axis optical scales and a digital readout; - Gap; - 3-jaw selfcentering chuck – 1 pc; - 4-jaw chuck – 1 pc; - Additional sets of jaws for 3-jaw chuck, straight and reversible, 1 set of each type; - Additional sets of jaws for 4-jaw chuck – 1 set; - Steady rest – 1 pc; - Travelling rest – 1 pc; - Thrust center – 1 pc; - Revolving center – 1 pc; - End stop. |
| 8. | # Delivery, training, launching | Equipment delivery (DAP, Incoterms 2010), personnel training, installation and commissioning of the equipment, and consumables (cables, wires and other materials, equipment, products) that will require equipment installation and start-up must be included in the basic price. |
| 9. | # Documentation | In the package of equipment documentation must be included operation and maintenance manuals, repair troubleshooting manuals with list of parts and components to be replaced and a list of operation codes, electrical, kinematic and other diagrams. Language of technical documentation – Russian and Latvian. |
| 10. | # Conformity to EU standards | Conformity to EU safety standards and CE marking |
| 11. | # Warranty | The warranty period is not less than 12 months after launching day. |